Changelog Firmware v.307_IP1000A

for DAS-3224/3248  HW rev.A1

Bug Fixed:

1. Fix download fails. In this problem download command was

showing error after downloading the complete.

2. Fix GVRP and GARP bug. Reboot after commit fails when GVRP and GARP are enabled  in the system and gvrpstatus is enable for a  port.

3. Fix IGMP group deleting.

Multicast Group deletion occurs at a regular interval in the

following scenario:

1. Keep sending IGMPv2 reports @ 1 packet/10second.

2. Keep sending IGMPv2 general queries @ 1 packet/10second.

3. Keep Max response time as 125 in queries.

4. keep a gap of 5-6s between a query and a report.

In this scenario after receiving 15 queries for that group

gets deleted and then it gets learned when next report comes

for that group. For this multicast data get dropped during

each deletion and learning of a group.

4. Fix IGMP leave.

If leave reports were sent after join for same multicast

Address from multiple (at least 7 or 8) ports and if we repeat

This for several times then system goes to High priority CLI

mode.

5. Fix  IGMP Port Register.

IGMP shows inconsistent behavior depending upon the order in

Which per port and global igmpsnoop was made enable in the

system. Here is how it occurs.

1. If igmpsnoop is disabled in the system but enabled per port

Then all the port got registered with packet filter.

2. If now IGMP packets are received then all the packets are

Delivered to igmpsnoop module and filter rule stats is

Increased. But it drops those packets as igmpsnoop is

Disabled in the system but it does not delete the frame

Allocated for those packets. Thus it causes a net buffer leak.

Once igmpsnoop is enabled and then set it to disable, filter

Rule stats do not increase.

6. Fix WHIP Trace.

Fix to solve the trace problem during commit reboot in case

WHIP running on.

7. Fix Dsl issues in Dataplane

Fix various dsl issues like:

a. Increase the DSLME GAG message queue size to avoid the message

drop issue if all lines are going down and coming up simultaneously.

b. Reduce DSLME trap burst size to avoid message overflow on GAG

Trap queue.

c. Dataplane fix to get fast response if all VLANs are created and having

one member from each ATM port.

8. Fix MDB Memory Allocated.

Dynamic Memory Allocated to MDB module is under-subscribed in

System resource memory file and result in failure of dsl system creation if

number of DSL ports in dsl system config is made same as given in

Manuf.txt. This patch is not meant for Telnet session issues.

9. Fix Apply Crash.

Fix the crash when we use telnet for apply command and

cfg file does not exist.

10. Fix Generic Filter compilation bug.

Fix the problem of internal system error when generic filter and

Classifier is compiled with optimization.

11. Fix Mdb Boundary Condition.

Solve the issue of boundary condition in MDB module for

getting mcast forwarding entry for max bridge port id. It will

display max. bridge port id properly.

12. Patch Telnet Inactivity Timeout. To provide this fix a new timeout variable

and validation for telnet timeout needs to be add in the existing code.

13. Fix Dsl Gag Trap Loss bug.

Increase GAG, DSLME queue size to avoid trap loss and

also introduce delay in meal to reduce delay in subsequent call

of HandleExtEvent() because of API was missing some traps from DSP.

14. Fix Trap log issue. The fix is related with the trap logging in flash. Logs get corrupted

when trap log block in flash got full and flash logging again starts

from top of flash block.

15. Fix G24 chip ID bug. System can know new G24 chip ID, 0xE8.

16. Fix Dsl Auto Reset. Patch to fix that DSL would autoreset if the led status and current   status are different.

17. Fix Light Dsl led each time.  Fix that DSL led doesn't be lighted up occasionally.

Various change:

1. Remove some adsl line profile paremeters.

In new firware version the four parameters bellow is removed from

"modify adsl line profile" command

- AtucGsStandard

- AtucGsAnnexType

- AtucGsAdvertisedCapabilities

- LineDmtConfMode

But replaced with following command

"modify adsl line intf ifname dsl-x LineTransAtucConfig <val set>

e.g. if you want to use Standard from gDmt to Adsl2Plus then use following

command, modify adsl line intf ifname dsl-x LineTransAtucConfig

q9925Adsl2PlusPotsNonOverlapped q9925Adsl2PlusPotsOverlapped

2. Support 4M flash

3. CLI timeout: 600 sec, Telnet without login: 60 sec

4. Add cli max session number to 8

5. Move log from sdram back to flash

6. Remove manuf.txt from flash

7. Dsl Counter

Feature Enhancement

- Support IGMP v3 and source multicast filtration (modify igmpsnoop mvlan config)

- Add few IGMP parameters (Igmpversionmask, maxgrpallowed, startupqrycount, lastmemberqrycount).