There are six files and one sub-directory (LPsource) in the utility

diskette.  The file names of the six files are:

1.readme.doc

2.doc.ps

3.dumb_int.sh

4.install.sh

5.instmsgs

6.mycc

--readme.doc

This file.

--doc.ps

This is a PostScript data file containing 04 ASCII codes at its beginning and

the end.  It is used for testing the PostScript printer connected with

Print_Server.

--dumb_int.sh

dumb_int.sh is a Bourne Shell script.  It is an interface program of a dumb

printer for System V Unix.  It pipes the banner page and data files to

PSfilter program.

--install.sh

install.sh is provided to help user attach a printer on Print_Server to the

UNIX system so that the printer acts as a local printer of the UNIX host.

install.sh attaches one printer at a time.  If you have more than one printer

connected to Print_Server, you must run install.sh several times.

If no paramaters are passed with install.sh, the install.sh program will go

through the interactive procedure, which will ask the questions about the

configuration of the printer one by one.

You can install Print_Server in a quick way with the default configuration

by passing the necessary arguments to install.sh.  The install.sh program for

quick setting takes the following arguments and options:

install.sh printer_name PS_HostName [pn=port_number] [im=interface_model]

[d=delay_minutes] [text]

where printer_name is the name of the printer to be added.

PS_HostName is the host name of the print server device.

interface_model is used only on system V, it is a model of the interface

program of the printer.

port_number can be one of the following, depending on your Print_Server model:

1               parallel port 1

2               parallel port 2

3               serial port

delay_minutes is the number of minutes allowed for printer faults before

the print job being aborted.  If delay_minutes is 0, the print job

will be aborted immediately after printer faults.  If it is -1, the

PSfilter program will keep waiting for the recovery of the printer

faults.

text is to instruct PSfilter to convert LineFeed to CarriageReturn + LineFeed.

port_number, interface_model, delay minutes and text mode are optional.

If they are not specified, the parallel port 1,  dumb_int.sh, -1(infinite

delay minutes) and non_text mode will be used.

--instmsgs

It contains all output messages of install.sh

--mycc

The mycc program is used to compile the psfilter.c source code.  Its output

execution file is PSfilter in the subdirectory ./LPsource.  To run mycc, type:

./mycc

It will be invoked by install.sh if there is no PSfilter in ./LPsource

directory.

LPsource subdirectory

---------------------

The LPsource subdirectory contains the following files:

--PS.sco, PS.sun5

These two executable files are provided in case there is no C Compiler in your

SCO 3.x or SUN 5.x UNIX system.  To use any of these two programs, you need to

copy the program file to a new file called "PSfilter" in the LPsource directory.

--psfilter.c, errormsg.h

psfilter.c & errormsg.h are the source files used to generate the PSfilter

executable program.

PSfilter resides in the LPsource directory by default.  PSfilter can be used as

a command line utility to print the print jobs through Print_Server.  PSfiler can

also be invokded by UNIX's lp deamon in the background.  PSfilter provides many

options to meet the needs of printing a print job.  PSfilter takes the following

options:

-D destination  Send data file to destination.  Destination is the host name

of Print_Server.  This option must be specified.

-S              Data is sent to serial port of Print_Server.

-P              Data is sent to parallel port 2 of Print_Server.

(If -S and -P are not specified, parallel port 1 will be

selected.)

-p string       Send string before a print job to initialize the printer.

If blank spaces are in the string, quotes must be used to

identify the string.

-s string       Send string after a print job to reset the printer.

If blank spaces are in the string, quotes must be used to

identify the string.

-h head_file    Send head_file before a print job to initialize the printer.

Head file must be specified with full path.

-t tail_file    Send tail_file after a print job to reset the printer.

Tail file must be specified with full path.

-d delay_min    The number of minutes allowed for the occurrence of the printer

faults before the active print job is aborted. (Default value of

delay minutes is 0.)

-a AnotherPort  Automatically send the print job to AnotherPort if the selected

printer is busy.

AnotherPort can be:

para1           Parallel port 1.

para2           Parallel port 2.

serial          Serial port.

all             The other two ports.

-b              If the network traffic is very heavy.  PSfilter will try more

times and wait longer than usual to do the job.

-T              Convert LF in the print job to CR/LF.

Note: If both pre_string and head_file are specified, the pre_string is sent

before the head_file.  Similarly, the tail file is sent before the

suf_string.

With the following commands, you can send a file to the parallel port 1,

parallel port 2 or serial port respectively if the hardware of Print_Server is

installed and its IP address is set.

PSfilter -D PS_name < file_name

PSfilter -D PS_name -P < file_name

PSfilter -D PS_name -S < file_name

********************* How to use Print_Server in a quick way ******************

1. Connect print cables and network cable to your Print_Server.

2. Connect power adapter to your Print_Server to power up the Print_Server.

3. Set the IP address of the Print_Server by rarp or telnet.

4. Go to the directory that the utilities will be loaded.  Insert the

utility diskette into your diskette device and type the command:

tar xvf device_name

5. Change to the ./LPTI directory created by the previous command and execute

install.sh.

cd LPTI

./install.sh  <parameters for quick setting or none>

6. On system V, print files by the following command:

lp -d printer_name  file_names

On BSD system , print files by the following command:

lpr -P printer_name  file_names

*** What Will You Do If You Failed To Use install.sh In A Quick Way

*** For Non-supported Unix Type To Install PSfilter

There is a chance that you may fail to use install.sh to install PSfilter

to your Unix host due to Unix type in your Unix system is not recognized

by install.sh. If you fail to use install.sh to install PSfilter to your

Unix host, type the following command to verify Unix type of your Unix

system:

uname [Enter]

install.sh can only recognize the following Unix types for the various

type of Unix:

scosysv                 for SCO Unix System V release 3

HP_UX                   for HP Unix

SunOS                   for Sun OS 4.x or 5.x on Sparc

UNIX_SV                 for AT&T unix_sv Release 4

OSF1                    for DEC/OSF1

If the response of the uname command does not match with the above mentioned

Unix types, there are three ways to provide solutions:

1. Modify the Unix type in install.sh to match with the Unix type checked

from uname command:

a) Make a copy of the install.sh file:

cp install.sh install.old

b) Edit install.sh file:

vi install.sh

c) There is a "check_unix_type" procedure between line 122 and 188 in

install.sh version 1.12. Modify the string of the Unix type

corresponding to your Unix type got from uname command.

Example:

The OS of your Unix is SCO Unix 3.2 but the Unix type got from

uname is HPTCP. In this case, modify the line of install.sh program,

scosysv ) unix_type = SCO

to

HPTCP ) unix_type = SCO

Note: Since Unix system is case sensitive, be careful of the letter

case when doing the modification.

2. Modify your Unix type to the type that install.sh recognize by using

uname command:

uname -S new_unix_type

Example:

The OS of your Unix is SCO Unix 3.2 but the Unix type got from uname

is HPTCP. In this case, type the following command:

uname -S scosysv

run the install.sh program again. After PSfilter has been installed

successfully, use the following command to change to Unix type back

to original:

uname -S HPTCP

3. Run the install.sh without any parameters by just typing:

./install.sh [Enter]

you will go through a question and answer process to input some

information to install.sh for it to do the installation.