# New Firmware Upgrade Procedure

## Users have to upgrade three different firmware files (fwu.hex, wssldr.hex, fwn.hex) in order to accomplish new firmware (v4.10.02) upgraded. Users can upgrade firmware through SmartConsole Utility or Web GUI as following steps:

File Requirement:

|  | DGS-1216T v.D1 | DGS-1224T v.D1 |
| --- | --- | --- |
| fwu.hex | 1216TFWU.hex | 1224TFWU.hex |
| wssldr.hex | 1216Tldr.hex | 1224Tldr.hex |
| fwn.hex | 1216TFWN.hex | 1224TFWN.hex |

1. Upgrade firmware through SmartConsole Utility (Please refer to below step. i-xi)

## Execute smart console utility

## Click Discovery button to search target switch

## Double click target switch to add to monitor list

## Click firmware upgrade button

## Click Browse button to search fwu.hex firmware file from user’s local PC

## Enter password and click start button to start firmware upgrade

## Wait for firmware upgrade complete

## Repeat step.i ~ step.vii, but choose WSSldr.hex to upgrade in step.v

## Repeat step.i ~ step.vii , but choose fwn.hex to upgrade in step.v

## 2. Upgrade firmware through Web GUI (Please refer to below step. i-vi)

## Open web GUI , clicking tool and choosing Firmware Backup & Upload

## Click browse button and choose fwu.hex firmware file from local user’s PC

## Click Upload button to start upgrade firmware

## iv.  Wait for firmware upgrade complete then restart Web GUI

## v.  Repeat step.i ~ step.iv , but choose WSSldr.hex in step.ii

## Repeat step.i ~ step.iv , but choose fwn.hex in step.ii