DWL-AG660

Date: March 15, 2004

| Product name: | 802.11a/g Wireless Cardbus Adapter |
| --- | --- |
| Product Group & Type: |  |
| Revision: | B1 |
| Available: |  |
| Picture: |  |
| Why Sell This Product: | - Super AG 108Mbps high speed dualband. This DWL-AG660 adapter will eventually replace the current DWL-AG650 adapter that supports only 54Mbps speed. The DWL-AG660's Super AG 108Mbps speed is provided by means of a software driver provided on the CD. Selling Points - Same price as DWL-AG650 but has higher speed. - Tri-mode a/b/g, dualband wireless. - Advanced security with WPA, 802.1x authentication, TKIP and Pre-Shared Key function support. |
| Keyfeatures: | - 802.11a/g industry standard - Dual 5GHz and 2.4GHz frequency bands - Up to 108Mbps (Turbo mode) network data transfer rate on both frequency bands, 15 times faster than 802.11b - Strong 64/128/152-bit WEP data encryption security - Compatible with existing 802.11b wireless devices - Advanced security with WPA, 802.1x authentication, TKIP and Pre-Shared Key function support - Powerful integrated antenna provides 100-meter indoor, - 400-meter outdoor operation range * - Runs Windows 98 SE, 2000, ME, XP * Environmental factors may adversely affect range. |
| Technical Details: | System Requirements - Laptop with available CardBus Type II slot - Windows XP, 2000, ME or 98 SE - 300MHz CPU or above - 32MB memory or more Standards - IEEE 802.11a - IEEE 802.11b - IEEE 802.11g PC Card Standard & Interface PC Card Type II CardBus 32-bit compliant Media Access Control CSMA/CA with ACK Data Rates (With automatic fallback) 54Mbps, 48Mbps, 36Mbps, 24Mbps, 18Mbps, 12Mbps, 11Mbps, 9Mbps, 6Mbps, 5.5Mbps, 2Mbps, 1Mbps Modulation Technology - Orthogonal Frequency Division Multiplexing (OFDM) - Complementary Code Keying (CCK) - Direct Sequence Spread Spectrum (DSSS) Frequency Range * - 2.4GHz to 2.5GHz - 5.15GHz to 5.875GHz * Software configurable as allowable frequency will vary from country to country in the European region. Transmit Output Power - 802.11a: +10 to 14dBm @ 54Mbps +11 to 15dBm @t 48Mbps +13 to 15dBm @t 36Mbps +17 to 18dBm @ 24, 18, 12, 9 and 6Mbps - 802.11b: +18dBm @ 11, 5.5, 2, and 1Mbps - 802.11g: +10 to 14dBm @ 54Mbps +11 to 15dBm @ 48Mbps +13 to 15dBm @ 36Mbps +17 to 18dBm @ 24, 18, 12, 9 and 6Mbps Receive Sensitivity - 802.11a (@ 10% PER): 87dBm for 6Mbps 86dBm for 9Mbps 85dBm for 12Mbps 83dBm for 18Mbps 80dBm for 24Mbps 76dBm for 36Mbps 71dBm for 48Mbps 66dBm for 54Mbps 73dBm for 108Mbps (Turbo Mode) - 802.11b (@ 8% PER): 84dBm for 11Mbps 90dBm for 2Mbps - 802.11g (@ 10% PER): 87dBm for 6Mbps 86dBm for 9Mbps 85dBm for 12Mbps 83dBm for 18Mbps 80dBm for 24Mbps 76dBm for 36Mbps 71dBm for 48Mbps 66dBm for 54Mbps Antenna 2dBi to 4dBi Gain integrated antenna Operation Range - Indoors: up to 100 meters * - Outdoors: up to 400 meters * * Environmental factors may adversely affect range. Security - 64/128/152-bit WEP data encryption - 802.1x authentication - WPA (Wi-Fi TM Protected Access): 802.1x, Pre Share Key, WEP, TKIP, AES Physical & Environmental LED Indicators - Power - Network Operating Voltage 5 VDC +/- 10% Power Consumption - Transmit mode: 1947mW - Receive mode: 1056mW - Sleep mode: 40mW Dimensions 118 (L) x 53 (W) x 5 (H) mm (including antenna) 84 (L) x 53 (W) x 3 (H) mm (adapter only) Weight 41grams (adapter including antenna) Operating Temperature 0 to 45 C Storage Temperature -10 to 60 C Humidity 95% maximum (non-condensing) EMI Certification - FCC Class B - CE Class B O.S. Support Windows 98 SE, ME, 2000, XP |
| EMI/EMC Report | FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No. ACN052202838  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others |
| Safety |  TUV No.  UL No.  CSA No.  Others |
| Packaging Style |  Clamshell  White Box  Gift Box  Bulk  Others |
| Packaging Contents: | 1. DWL-AG660 wireless Cardbus Adapter 2. Driver and Manual on CD-ROM 3. Quick Installation Guide |
| UPC Code: | DWLAG660EUB1 790069 265228; DWLAG660FRB1 790069 265235; DWLAG660SGB1 790069 265242. DWLAG660AUB1 790069 265259; DWLAG660LAB1 790069 265266; TDWLAG660.B1 790069 265280. |
| Units Per Carton: | 40 |
| Cartons Per Pallet: | Air: 18, Sea: 30 |
| Dimensions and Weight: | Box: 237 x 170 x 35 mm, 0.35 kg Carton: 385 x 352 x 502 mm, 17.7 kg Pallet: (by Air) 1200 x 1100 x 1260 mm, 274 kg (by Sea) 1200 x 1100 x 2020 mm, 447 kg INTRASTAT weight: kg (Box) Packing material weight (Box): - Paper: kg - Plastic: kg - Mixed: kg |